REPLACING AN SMC

When replacing an SMC module, make the SMC that you are NOT replacing the ACTIVE SMC.

Pull the SMC and swap out the flash cards.

Once the SMC is replaced and the SMC is in Standby mode, use the command - show hd raid verbose – and see if Raid is degraded.

If Raid is degraded, the right hand side of the illustrations below will most likely show the same symptoms that you are encountering.

| Both hard disks are good: Degraded: no In-sync | Hard disk on SMC8 is not being use. “Degrated” show yes. “In-sync component” is smc9 only |
| --- | --- |
| [local]CHRXNCLHPN3 HA# show hd raid verbose HD RAID: State : Available (clean) Degraded : No UUID : b9594ac4:0f3f1061:6f0cff7a:7f25676d Size : 146000000000 bytes Action : Idle Disk : hd-local1 State : In-sync component Created : Fri May 15 06:06:19 2009 Updated : Tue Apr 27 18:33:36 2010 Events : 134 Model : ********************* Serial Number : ********************* Location : SMC8 PLB46075595 Size : 146815737856 bytes Partitions : 1 Partition 1 : 146006913024 bytes or 285169752 sectors Disk : hd-remote1 State : In-sync component Created : Fri May 15 06:06:19 2009 Updated : Tue Apr 27 18:33:36 2010 Events : 134 Model : ********************* Serial Number : ********************* Location : SMC9 PLB46075623 Size : 146815737856 bytes Partitions : 1 Partition 1 : 146006913024 bytes or 285169752 sectors | [local]WNDSCTWLPN4 HAGR# show hd raid verbose HD RAID: State : Available (clean) Degraded : Yes UUID : 832f662f:3081f6b1:a73b6788:8c0a031c Size : 146000000000 bytes Action : Idle Disk : hd-local1 State : Valid image of c31c353c:e728917c:a8f9f7c9:d0f07cd2 Created : Fri Oct 17 16:57:24 2008 Updated : Fri Oct 24 18:46:43 2008 Events : 11973 Model : ********************* Serial Number : ********************* Location : SMC8 PLB30085891 Size : 146815737856 bytes Partitions : 1 Partition 1 : 146006913024 bytes or 285169752 sectors Disk : hd-remote1 State : In-sync component Created : Mon Jan 5 18:19:05 2009 Updated : Tue Apr 27 18:31:47 2010 Events : 7539494 Model : ********************* Serial Number : ********************* Location : SMC9 PLB41088233 Size : 147086327808 bytes Partitions : 1 Partition 1 : 146006913024 bytes or 285169752 sectors |

The hard drive that is out of sync with Raid will show “Valid image of” in the State field, from the command show hd raid verbose.  This is seen above, on the right hand side pane.

To rectify Raid being out of sync, use the command - hd raid overwrite remote1 or hd raid overwrite local1 .  In the case above, the command hd raid overwrite local1 should be used.

It will take an estimated 30-40 minutes for Raid to become synchronized.  If you want to watch Raid becoming synchronized – which you must at some point - you can either enable debug logging for the hdctrl process, or keep issuing the command – show hd raid verbose.

Once Raid and the hard drives are synchronized, check the infrastructure of the Phy links between the two hard drives by using the command - debug hdctrl lssas  .

An example of the output from this command is below. Also check the hard drives using the hidden command:  show hd smart local1 or show hd smart remote1.

The smart tests should be running nothing abnormal should be seen.  If there is a suspected issue, open a case with engineering and get their input.

Check to make sure that FDR’s are being written to the hard drive and the L-ESS server.  You can use the command:  dir /hd-raid/records/edr

And re-issue the command to make sure billing records are being written, and then moved.  Also check show active-charging edr-udr-file statistics, looking for Files or Records deleted, and anything else applicable.

From the command - debug hdctrl lssas -, focus your attention on the Phy 4 and Phy 5 links on both SMC 8 and SMC 9.

These are interconnects which provide connectivity to each of the hard drives, and backups to each other.

If one (1) Phy has a connectivity issue (broken link), you will see one of the Phy’s reporting:

PHY 5 [ , no link, no port]       <=== MISSING, broken

If this is the case, and one of the links are broken, at this point I consider it safe to issue the command:

hd raid reset-phy {local1|remote1} – which holds the possibility of resetting the failed Phy link.

Check debug hdctrl lssas again.  If one Phy link is still broken, you can try to resolve this problem by physically removing and then re-inserting the SMC.

If this does not resolve the Phy link issue, some consideration as to what Raid and the hard drives house (billing records) must be given before proceeding.  If there is one Phy link broken, check the amount of time left in the maintenance window. If you have as a minimum two hours available, then it is advisable to move the subscribers to the HAGR before continuing.  Make sure there are no billing records remaining on the SMC’s hard drive (dir /hd-raid/records/edr).  If they are, use the command cdr-push all.

Make sure the HAGR is stable at this time.

At this point kill the Raid Array using the hidden command:

debug hdctrl shutdown

Format the hard drives – both of them together.  Enable logging using the following commands:

logging active

logging filter active facility hdctrl level info

The next commands will take an estimated 1 hour.

hd raid format local1

hd raid format remote1

Once completed, use the following command to recreate the Raid Array

debug hdctrl restart

Check the Raid array using the command show hd raid verbose

Check the Phy links using debug hdctrl lssas

Check show hd smart local1 or show hd smart remote1

Check show active-charging edr-udr-file statistics

Check dir /hd-raid/records/edr

Check the license on the chassis.

If there are further problems, such as for instance with one of the Phy links, you will have to decide whether or not to move the subscribers back to the HA before opening a case with engineering.

One last command, which has little to do with swapping out the SMC’s:

If you are called one day/night for a billing record issue and find that the Raid is down completely, and the hard drives are not responding, you can issue the command:

hd raid reset-dev {local1|remote1}      which may reset an unresponsive disk

Additional link to the associated wiki below:

http://wikicentral.cisco.com/display/GROUP/How+to+recreate+RAID+that+was+out+of+sync

******** debug hdctrl lssas *******

Local card (SMC8):
vsc725x rev D at 0000:06:00.0
 SCSI host 0 [Vitesse VSC7250 SAS/SATA Host Adapter, 5000547000000701]
   PHY 0 [SAS initiator, no link, no port]
   PHY 1 [SAS initiator, no link, no port]
   PHY 2 [SAS initiator, no link, no port]
   PHY 3 [SAS initiator, no link, no port]
   PHY 4 [SAS initiator, 3.0 Gbps, port 0]
     Domain 5000c50003a395a5 [SSP end device]
       LUN 0000000000000000 [channel 0]
         SCSI target 0:0:0:0
           Vendor SEAGATE 
           Model  ST9146802SS     
         Device sda [dev=8:0, 146815737856 bytes (136 GB)]
           Partition sda1 [dev=8:1, 146006913024 bytes (135 GB)]
   PHY 5 [SAS initiator, 3.0 Gbps, port 1]
     Domain 5000c500054b780a [SSP end device]
       LUN 0000000000000000 [channel 1]
         SCSI target 0:1:0:0
           Vendor SEAGATE 
           Model  ST9146802SS     
         Device sdb [dev=8:16, 146815737856 bytes (136 GB)]
           Partition sdb1 [dev=8:17, 146006913024 bytes (135 GB)]
   PHY 6 [SAS initiator, no link, no port]
   PHY 7 [SAS initiator, no link, no port]


Remote card (SMC9):
vsc725x rev D at 0000:06:00.0
 SCSI host 0 [Vitesse VSC7250 SAS/SATA Host Adapter, 5000547000000801]
   PHY 0 [SAS initiator, no link, no port]
   PHY 1 [SAS initiator, no link, no port]
   PHY 2 [SAS initiator, no link, no port]
   PHY 3 [SAS initiator, no link, no port]
   PHY 4 [SAS initiator, 3.0 Gbps, port 0]
     Domain 5000c500054b7809 [SSP end device]
       LUN 0000000000000000 [channel 0]
         SCSI target 0:0:0:0
           Vendor SEAGATE 
           Model  ST9146802SS     
         Device sda [dev=8:0, 146815737856 bytes (136 GB)]
           Partition sda1 [dev=8:1, 146006913024 bytes (135 GB)]
   PHY 5 [ , no link, no port]                     <=== MISSING, broken
   PHY 6 [SAS initiator, no link, no port]
   PHY 7 [SAS initiator, no link, no port]