https://supportforums.cisco.com/message/4178990#4178990

cisco

VLAN1 interface 192.168.2.254

Create a second VLAN, in this case, we create VLAN 2 with the name Internet

Assign a static IP address to the second VLAN you've created, in this case we will be using 192.168.3.254 on a /24 mask for the VLAN 2

Assign the switch ports you wish to be a member of the VLAN you created, in this case, I have assigned port 2 to VLAN 2 untagged

- Enable the DHCP server on the switch

Create a DHCP pool, in this case, we will be creating the pool to correlate to VLAN 2 on the 192.168.3.x subnet

Log in to your RV0XX router and enable multiple subnet

Define your multiple subnet, in this example, 192.168.3.0 network with /24 mask

This step in on a Cisco  Router

For Pfsense firewall router see end of document

Create a static route to direct traffic to 192.168.3.0 subnet to the VLAN 1 gateway, on this case it is 192.168.2.254

Remember we configured port 2 as VLAN 3 untagged. Move a connection to port 2 and verify you receive a proper DHCP address and confirm the internet works

Create a second subnet 192.168.10.1/24 on the same LAN2 interface (192.168.2.1)

It is call Virtual IP Alias

Pfsense -> Virtual IP

Modifying the firewall rules on LAN2  Source = Any Destination = Any or add another rule specific to the VIP Alias

Adding to Firewall NAT outbound

Testing

VLAN10 should be able to access the internet

VLAN10 should be able to see VLAN1

CAN NOT Ping Virtual IP 192.168.10.1

CAN PING Physical IP 192.168.2.1